Written Program

for

# Respiratory Protection

(29 CFR 1910.134)

Respiratory Protection

Table of Contents

Objective

Assignment of Responsibility

  - Employer

  - Program Administrator

  - Supervisors

  - Employees

Applicability

Program

  - Hazard Assessment and Respirator Selection

  - Updating the Hazard Assessment

  - Training

  - NIOSH Certification

  - Voluntary Respirator Use

  - Medical Evaluation

  - Fit Testing

  - General Respirator Use Procedures

  - Change Schedules

  - Cleaning

  - Maintenance

  - Storage

  - Respirator Malfunctions and Defects

  - Program Evaluation

  - Documentation and Recordkeeping

Attachments

  - Sample Hazard Assessment Log

  - Sample Record of Respirator Use

  - Sample Hazard Evaluation

  - Sample Record of Respirator Issuance

  - Respirator Inspection Checklist

  - Sample Emergency Potential Log

  - Sample IDLH Assessment

  - Sample Company Facial Hair Policy

  - Respirator Audit form

  - Voluntary Use form

  - Sample Fit Test Record

  - Definitions

- Respiratory Protection Program

- OBJECTIVE

- The ___(Company Name)____ Respiratory Protection Program is designed to protect employees by establishing accepted practices for respirator use, providing guidelines for training and respirator selection, and explaining proper storage, use and care of respirators.  This program also serves to help the Company and its employees comply with Occupational Safety and Health Administration (OSHA) respiratory protection requirements as found in 29 CFR 1910.134.

- ASSIGNMENT OF RESPONSIBILITY

  - Employer

- The Company is responsible for providing respirators to employees when they are necessary for health protection, and it will provide respirators that are applicable and suitable for the intended purpose at no charge to affected employees.  Any expense associated with training, medical evaluations and respiratory protection equipment will be borne by the Company.

  - Program Administrator

- The Program Administrator for the Company is (Designated Person’s Name).  The Program Administrator is responsible for administering the respiratory protection program. Duties of the program administrator include:

    - Identifying work areas, process or tasks that require workers to wear respirators.

    - Evaluating hazards.

    - Selecting respiratory protection options.

    - Monitoring respirator use to ensure that respirators are used in accordance with their specifications.

    - Arranging for and/or conducting training.

    - Ensuring proper storage and maintenance of respiratory protection equipment.

    - Conducting quantitative or qualitative fit testing per the requirements of the OSHA Respiratory Protection Standard, 1910.134 Appendix A.

    - Administering the medical surveillance program.

    - Maintaining records required by the program.

    - Evaluating the program’s effectiveness.

    - Updating written program, as needed.

  - Supervisors

Supervisors are responsible for ensuring that the respiratory protection program is implemented in their particular areas.  In addition to being knowledgeable about the program requirements for their own protection, supervisors must also ensure that the program is understood and followed by the employees under their charge.  Duties of the supervisor include:

    - Ensuring that employees (including new hires) receive appropriate training, fit testing, and annual medical evaluation.

    - Ensuring the availability of appropriate respirators and accessories.

    - Being aware of tasks requiring the use of respiratory protection.

    - Enforcing the proper use of respiratory protection when necessary.

    - Ensuring that respirators are properly cleaned, maintained, and stored according to the standard (1910.134).

    - Ensuring that respirators fit appropriately and do not cause discomfort.

    - Continually monitoring work areas and operations to identify respiratory hazards.

    - Coordinating with the Program Administrator to address respiratory hazards or other concerns regarding this program.

  - Employees

Each employee is responsible for wearing his or her respirator when and where required and in the manner in which they are trained.  Employees must also:

    - Care for and maintain their respirators as instructed, guard them against damage, and store them in a clean, sanitary location.

    - Inform their supervisor if their respirator no longer fits well, and request a new one that fits properly.

    - Inform their supervisor or the Program Administrator of any respiratory hazards that they feel are not adequately addressed in the workplace and of any other concerns that they have regarding this program.

    - Use the respiratory protection in accordance with the manufacturer’s instructions and the training received.

- APPLICABILITY

This program applies to all employees who are required to wear respirators during normal work operations, as well as during some non-routine or emergency operations, such as a spill of a hazardous substance.

The OSHA Beryllium standard (1910.1024) identifies specific requirements for respiratory protection (1910.1024 (g); the requirements have been reprinted as follows from the standard:

- General.  The employer must provide respiratory protection at no cost to the employee and ensure that each employee uses respiratory protection:

  - During periods necessary to install or implement feasible engineering and work practice controls where airborne exposure exceeds, or can reasonably be expected to exceed, the TWA PEL or STEL;

  - During operations, including maintenance and repair activities and non-routine tasks, when engineering and work practice controls are not feasible and airborne exposure exceeds, or can reasonably be expected to exceed, the TWA PEL or STEL;

  - During operations for which an employer has implemented all feasible engineering and work practice controls when such controls are not sufficient to reduce airborne exposure to or below the TWA PEL or STEL;

  - During emergencies; and

  - When an employee who is eligible for medical removal under paragraph (l)(1) chooses to remain in a job with airborne exposure at or above the action level, as permitted by paragraph (l)(2)(ii) of this standard.

- Respiratory protection program.  Where this standard requires an employer to provide respiratory protection, the selection and use of such respiratory protection must be in accordance with the Respiratory Protection standard (§ 1910.134).

- The employer must provide at no cost to the employee a powered air-purifying respirator (PAPR) instead of a negative pressure respirator when;

  - Respiratory protection is required by this standard;

  - An employee entitled to such respiratory protection requests a PAPR; and

  - The PAPR provides adequate protection to the employee in accordance with paragraph (g)(2) of this standard.

- PROGRAM

  - Hazard Assessment and Respirator Selection

The Program Administrator will select respirators to be used on site, based on the hazards to which workers are exposed and in accordance with the OSHA Respiratory Protection Standard.  The Program Administrator will conduct a hazard evaluation for each operation, process, or work area where airborne contaminants may be present in routine operations or during an emergency.  A log of identified hazards will be maintained by the Program Administrator (See Sample Hazard Evaluation, Attachment C).  The hazard evaluations shall include:

    - Identification and development of a list of hazardous substances used in the workplace by department or work process.

    - Review of work processes to determine where potential exposures to hazardous substances may occur.  This review shall be conducted by surveying the workplace, reviewing the process records, and talking with employees and supervisors.

    - Exposure monitoring to quantify potential hazardous exposures.

The proper type of respirator for the specific hazard involved will be selected in accordance with the manufacturer’s instructions.  A list of employees and appropriate respiratory protection will be maintained by the Program Administrator (see Attachment D).

  - Updating the Hazard Assessment

The Program Administrator must revise and update the hazard assessment as needed (i.e., any time work process changes may potentially affect exposure).  If an employee feels that respiratory protection is needed during a particular activity, he/she is to contact his/her supervisor or the Program Administrator.  The Program Administrator will evaluate the potential hazard, and arrange for outside assistance as necessary.  The Program Administrator will then communicate the results of that assessment to the employees.  If it is determined that respiratory protection is necessary, all other elements of the respiratory protection program will be in effect for those tasks, and the respiratory program will be updated accordingly.

  - Training

The Program Administrator will provide training to respirator users and their supervisors on the contents of the Company’s Respiratory Protection Program and their responsibilities under it, and on the OSHA Respiratory Protection Standard. All affected employees and their supervisors will be trained prior to using a respirator in the workplace. Supervisors will also be trained prior to supervising employees that must wear respirators.

The training course will cover the following topics:

    - The Company Respiratory Protection Program;

    - the OSHA Respiratory Protection Standard (29 CFR 1910.134);

    - respiratory hazards encountered at the Company facilities, and their health affects;

    - proper selection and use of respirators;

    - limitations of respirators;

    - respirator donning and user seal (fit) checks;

    - fit testing;

    - emergency use procedures;

    - maintenance and storage; and

    - medical signs and symptoms limiting the effective use of respirators.

Employees will be retrained annually or as needed (e.g., if they change departments or work processes and need to use a different respirator). Employees must demonstrate their understanding of the topics covered in the training through hands-on exercises and a written test. Respirator training will be documented by the Program Administrator and the documentation will include the type, model, and size of respirator for which each employee has been trained and fit tested.

  - NIOSH Certification

All respirators must be certified by the National Institute for Occupational Safety and Health (NIOSH) and shall be used in accordance with the terms of that certification. In addition, all filters, cartridges, and canisters must be labeled with the appropriate NIOSH approval label. The label must not be removed or defaced while the respirator is in use.

  - Voluntary Respirator Use

The Program Administrator shall authorize voluntary use of respiratory protective equipment as requested by all other workers on a case-by-case basis, depending on specific workplace conditions and the results of medical evaluations.

The requirements are slightly relaxed when the employee voluntarily chooses to wear a respirator, where OSHA or the employer does not require such use.  Such circumstances typically include use of a dust mask when performing occasional dusty tasks, such as sweeping or outdoors work ( ex.- mowing grass, outdoor work during allergy season).

The respiratory protection standard addresses the three steps that the employer must take to prevent health hazards.  These steps are:

- A minimal training program to consist of providing Appendix D to the employee;

- Medical evaluation of fitness to wear a respirator; and

- A program to insure proper cleaning, storage, and maintenance of the respirator.

The Company may provide respirators to employees who request them but employees are NOT permitted to provide their own respirators for voluntary use and the employer may also limit or prohibit their use.

Note:	If the only respirators that are voluntarily used are filtering facepieces (ex. - 3M 8210 dust masks), the only required step is the provision of Appendix D to employees who choose to wear dust masks.  There are no medical limitations on the use of filtering facepiece respirators when worn in a voluntary situation where respiratory hazards do not exist.

In addition, any employee who voluntarily wears a respirator when one is not required is subject to the medical evaluation, cleaning, maintenance, and storage elements of this program, and will be provided with necessary training.

All employees and processes that fall under the provisions of this program are listed in Attachment D.

  - Medical Evaluation

Employees who are either required to wear respirators, or who choose to wear a half face piece APR voluntarily, must pass a medical exam provided by the Company before being permitted to wear a respirator on the job. Employees are not permitted to wear respirators until a licensed health care professional (PLHCP)  has determined that they are medically able to do so. Any employee refusing the medical evaluation will not be allowed to work in an area requiring respirator use.

A licensed health care professional (PLHCP)  will provide the medical evaluations required for respirator wearers as dictated by the OSHA Respiratory Protection Standard, 29 CFR 1910.134.  Medical evaluation procedures are as follows:

    - The medical evaluation will be conducted using the questionnaire provided in Appendix C of the OSHA Respiratory Protection Standard.  The Program Administrator will provide a copy to all employees requiring medical evaluations.  To the extent feasible, the company will provide assistance to employees who are unable to read the questionnaire.  When this is not possible, the employee will be sent directly to a physician for medical evaluation.

    - All affected employees will be asked to complete the medical questionnaire,. Employees will be permitted to complete the questionnaire on company time.

    - Follow-up medical exams will be granted to employees as required by the OSHA Respiratory Protection standard, and/or as deemed necessary by the evaluating physician.

    - All employees will be granted the opportunity to speak with the physician about their medical evaluation, if they so request.

    - The Program Administrator shall provide the evaluating physician with a copy of this Program, a copy of the OSHA Respiratory Protection Standard, the list of hazardous substances by work area, and the following information about each employee requiring evaluation:

      - his or her work area or job title;

      - proposed respirator type and weight;

      - length of time required to wear respirator;

      - expected physical work load (light, moderate or heavy);

      - potential temperature and humidity extremes; and

      - any additional protective clothing required.

    - Powered air purifying respirators (PAPR) will be provided to employees as required by medical necessity and as the beryllium standard 1910.1024 requires when respiratory protection is required due to airborne exposure above the PEL or STEL and the employee requests a PAPR.

    - After an employee has received clearance to wear his or her respirator, additional medical evaluations will be provided under the following circumstances:

      - The employee reports signs and/or symptoms related to their ability to use the respirator, such as shortness of breath, dizziness, chest pains or wheezing.

      - The evaluating physician or supervisor informs the Program Administrator that the employee needs to be reevaluated.

      - Information found during the implementation of this program, including observations made during the fit testing and program evaluation, indicates a need for reevaluation.

      - A change occurs in workplace conditions that may result in an increased physiological burden on the employee.

The PLHCP’s approval shall be a written certification that lists the respirator types approved for use by the individual (i.e. negative pressure air purifying, powered air purifying, pressure demand SCBA) and any restrictions on the employee’s use of respiratory protection.

The physician’s certification shall not disclose any confidential medical information, but shall clearly list or describe any restrictions to be observed.  A copy shall be provided to the EHS Department and Supervisor.

All examinations and questionnaires are to remain confidential between the employee and the physician.  The Program Administrator will only retain the physician’s written recommendations regarding each employee’s ability to wear a respirator. Medical records created under this program shall be handled in accordance with OSHA requirements for confidentiality, employee access and retention. All medical records shall be maintained by the Human Resources department.

A list of the Company employees currently included in medical surveillance is provided in Attachment D of this program.

  - Fit Testing

The Program Administrator shall arrange for fit tests in accordance with the OSHA Respiratory Protection Standard. Employees who are required to or who voluntarily wear half-face piece APRs will be fit tested:

    - prior to being allowed to wear any respirator with a tight-fitting face piece;

    - annually; or

    - when there are changes in the employee’s physical condition that could affect the respirator fit (e.g., obvious change in body weight, facial scarring, etc.).

    - where required by OSHA regulations

Employees will be fit tested with the make, model, and size of respirator that they will actually wear.  Employees will be provided with several models and sizes of respirators so that they may find an optimal fit.

  - General Respirator Use Procedures

    - Employees will use their respirators under conditions specified in this program, and in accordance with the training they receive on the use of each particular model.  In addition, the respirator shall not be used in a manner for which it is not certified by NIOSH or by its manufacturer.

    - All employees shall conduct user seal checks each time they wear their respirators.  Employees shall use both the positive and negative pressure check  as specified in the OSHA Respiratory Protection Standard.

      - Positive Pressure Test:  This test is performed by closing off the exhalation valve with your hand.  Exhale air into the mask.  The face fit is satisfactory if some pressure can be built up inside the mask without any air leaking out between the mask and the face of the wearer.

      - Negative Pressure Test:  This test is performed by closing of the inlet openings of the cartridge with the palm of your hand.  Some masks may require that the filter holder be removed to seal off the intake valve.  Inhale gently so that a vacuum occurs within the face piece.  Hold your breath for ten (10) seconds.  If the vacuum remains, and no inward leakage is detected, the respirator is fit properly.

    - All employees shall be permitted to leave the work area to maintain their respirator for the following reasons:

      - to clean their respirator if it is impeding their ability to work;

      - to change filters or cartridges;

      - to replace parts; or

      - to inspect respirator if it stops functioning as intended.

- Employees should notify their supervisor before leaving the area.

    - Employees are not permitted to wear tight-fitting respirators if they have any condition, such as facial scars, facial hair, or missing dentures that would prevent a proper seal.  Employees are not permitted to wear headphones, jewelry, or other items that may interfere with the seal between the face and the face piece.

    - Before and after each use of a respirator, an employee or immediate supervisor must make an inspection of tightness or connections and the condition of the face piece, headbands, valves, filter holders and filters.  Questionable items must be addressed immediately by the supervisor and/or Program Administrator.

  - Change Schedules

Respirator cartridges shall be replaced as determined by the Program Administrator, supervisor(s), and manufacturers’ recommendations.

  - Cleaning

Respirators are to be regularly cleaned and disinfected at the designated respirator cleaning station in accordance with the requirements set forth in the OSHA standard.  Respirators issued for the exclusive use of an employee shall be cleaned as often as necessary.  Atmosphere-supplying and emergency use respirators are to be cleaned and disinfected after each use.

- The following procedure is to be used when cleaning and disinfecting reusable respirators:

    - Disassemble respirator, removing any filters, canisters, or cartridges.

    - Wash the face piece and all associated parts (except cartridges and elastic headbands) in an approved cleaner-disinfectant solution in warm water (about 120 degrees Fahrenheit).  Do not use organic solvents.  Use a hand brush to remove dirt.

    - Rinse completely in clean, warm water.

    - Disinfect all facial contact areas by spraying the respirator with an approved disinfectant.

    - Air dry in a clean area.

    - Reassemble the respirator and replace any defective parts.  Insert new filters or cartridges and make sure the seal is tight.

    - Place respirator in a clean, dry plastic bag or other airtight container.

- The Program Administrator will ensure an adequate supply of appropriate cleaning and disinfection materials at the cleaning station.  If supplies are low, employees should notify their supervisor, who will inform the Program Administrator.

  - Maintenance

Respirators are to be properly maintained at all times in order to ensure that they function properly and protect employees adequately.  Maintenance involves a thorough visual inspection for cleanliness and defects. Worn or deteriorated parts will be replaced prior to use.  No components will be replaced or repairs made beyond those recommended by the manufacturer.  Repairs to regulators or alarms of atmosphere-supplying respirators will be conducted by the manufacturer.

    - All respirators shall be inspected routinely before and after each use.

    - Respirators kept for emergency use shall be inspected after each use, and at least monthly by the Program Administrator to assure that they are in satisfactory working order

    - The Respirator Inspection Checklist (Attachment E) will be used when inspecting respirators.

    - A record shall be kept of inspection dates and findings for respirators maintained for emergency use.

    - Employees are permitted to leave their work area to perform limited maintenance on their respirator in a designated area that is free of respiratory hazards.  Situations when this is permitted include:

      - washing face and respirator face piece to prevent any eye or skin irritation;

      - replacing the filter, cartridge or canister;

      - detection of vapor or gas breakthrough or leakage in the face piece; or

      - detection of any other damage to the respirator or its components.

  - Storage

After inspection, cleaning, and necessary repairs, respirators shall be stored appropriately to protect against dust, sunlight, heat, extreme cold, excessive moisture, or damaging chemicals.

    - Respirators must be stored in a clean, dry area, and in accordance with the manufacturer’s recommendations.  Each employee will clean and inspect their own air-purifying respirator in accordance with the provisions of this program, and will store their respirator in a plastic bag or other approved container in the designated area.  Each employee will have his/her name on the bag and that bag will only be used to store that employee’s respirator.

    - Respirators shall be packed or stored so that the face piece and exhalation valve will rest in a near normal position.

    - Respirators shall not be placed in lockers or toolboxes unless they are in carrying cartons.

    - Respirators maintained at stations and work areas for emergency use shall be stored in compartments built specifically for that purpose, be quickly accessible at all times, and be clearly marked.

    - The Program Administrator will store the Company’s supply of respirators and respirator components in their original manufacturer’s packaging in the (Area Name).

  - Respirator Malfunctions and Defects

    - For any malfunction of a respirator, such as breakthrough, face piece leakage, or improperly working valve, the respirator wearer should inform his/her supervisor that the respirator no longer functions as intended, and go to the designated safe area to maintain the respirator.  The supervisor must ensure that the employee either receives the needed parts to repair the respirator or is provided with a new respirator.

    - Respirators that are defective or have defective parts shall be taken out of service immediately.  If, during an inspection, an employee discovers a defect in a respirator, he/she is to bring the defect to the attention of his/her supervisor.  Supervisors will give all defective respirators to the Program Administrator.  The Program Administrator will decide whether to:

      - temporarily take the respirator out of service until it can be repaired;

      - perform a simple fix on the spot, such as replacing a head strap; or

      - dispose of the respirator due to an irreparable problem or defect.

- When a respirator is taken out of service for an extended period of time, the respirator will be tagged out of service, and the employee will be given a replacement of a similar make, model, and size.  All tagged out respirators will be kept in the (Area Name).

  - Program Evaluation

The Program Administrator will conduct periodic evaluations of the workplace to ensure that the provisions of this program are being implemented. The evaluations will include regular consultations with employees who use respirators and their supervisors, site inspections, air monitoring and a review of records.  Items to be considered will include:

    - comfort;

    - ability to breathe without objectionable effort;

    - adequate visibility under all conditions

    - provisions for wearing prescription glasses;

    - ability to perform all tasks without undue interference; and

    - confidence in the face piece fit.

Identified problems will be noted in an inspection log and addressed by the Program Administrator. These findings will be reported to the Company management, and the report will list plans to correct deficiencies in the respirator program and target dates for the implementation of those corrections.

  - Documentation and Recordkeeping

    - A written copy of this program and the OSHA Respiratory Protection Standard shall be kept in the Program Administrator’s office and made available to all employees who wish to review it.

    - Copies of training and fit test records shall be maintained by the Program Administrator.  These records will be updated as new employees are trained, as existing employees receive refresher training, and as new fit tests are conducted.

    - For employees covered under the Respiratory Protection Program, the Program Administrator shall maintain copies of the physician’s written recommendation regarding each employee’s ability to wear a respirator.  The completed medical questionnaires and evaluating physician’s documented findings will remain confidential in the employee’s medical records at the location of the evaluating physician’s practice.

ATTACHMENT A

Hazard Assessment Log

| Hazard Assessment Log M/D/Y |  |  |  |  |
| --- | --- | --- | --- | --- |
| Department | Contaminants | Exposure Level (8 hr TWA*) | OEL** | Controls |
|  |  |  | . |  |

*	Summarized from Industrial Hygiene report provided by a qualified professional.

**	The Occupational Exposure Limits observed for the Company’s airborne contaminants are; the OSHA Permissible Exposure Limit (PEL), the NIOSH Recommended Exposure Limit (REL), the American Conference of Governmental Industrial Hygienists (ACGIH) Threshold Limit Values (TLV)  and/or the applicable Industry’s exposure limit..

ATTACHMENT B

Record of Respirator Use

| Required and Voluntary Respirator Use at (Company Name) |  |
| --- | --- |
| Type of Respirator | Department/Process |

ATTACHMENT C

Hazard Evaluation

| Process Hazard Evaluation for (Company Name) M/D/Y |  |
| --- | --- |
| Process | Noted Hazards |

ATTACHMENT D

Record of Respirator Issuance

| (Company Name). Personnel in Respiratory Protection Program M/D/Y |  |  |  |  |
| --- | --- | --- | --- | --- |
| Respiratory protection is required for and has been issued to the following personnel: |  |  |  |  |
| Name | Department | Job Description/ Work Procedure | Type of Respirator | Date Issued |

ATTACHMENT E

Respirator Inspection Checklist

| Type of Respirator: | Location: |
| --- | --- |
| Respirator Issued to: | Type of Hazard: |
| Face piece | _________ Cracks, tears, or holes _________ Face mask distortion _________ Cracked or loose lenses/face shield |
| Head straps | _________ Breaks or tears _________ Broken buckles |
| Valves: | _________ Residue or dirt _________ Cracks or tears in valve material |
| Filters/Cartridges: | _________ Approval designation _________ Gaskets _________ Cracks or dents in housing _________ Proper cartridge for hazard |
| Air Supply Systems | _________ Breathing air quality/grade _________ Condition of supply hoses _________ Hose connections _________ Settings on regulators and valves |
| Rubber/Elastomer Parts | _________ Pliability _________ Deterioration |

| Inspected by: | Date: |
| --- | --- |
| Action Taken: |  |

ATTACHMENT F

Sample Emergency Potential Log

The following work areas at   (Company Name)    have been identified as having foreseeable emergencies:

| Area | Type of Emergency | Location of Emergency Respirator(s) |
| --- | --- | --- |

__________________________________ 	____________________

Program Administrator	    Date

ATTACHMENT G

Immediately Dangerous to Life and Health (IDLH) Assessment Log

The Program Administrator has identified the following areas as presenting the potential for IDLH conditions:

| Process | IDLH Condition | Procedure |
| --- | --- | --- |

___________________________________ 	____________________________

Program Administrator                                                       Date

ATTACHMENT H

Sample Facial Hair Policy

Company Facial Hair Policy

(Company) has adopted a Facial Hair Policy in accordance with the OSHA Respiratory Standard 1910.134.  The purpose of the policy is to provide the maximum protection possible to employees wearing respiratory protection.

The policy has been reviewed by _____________________ and endorsed by Management.  When a respirator is worn for protection against actual or potential airborne contaminants or for employee’s comfort in an area that does not require respiratory protection, the policy is as follows:

“Respirators shall not be worn when conditions prevent a good face seal.  A respirator shall not be worn if facial hair comes between the sealing peripheries of the face piece and the face or facial hair interferes with the function of the respirator.”  A PAPR (Powered Air Purifying Respirator) may be worn in place of a tight fitting Negative Pressure Respirator for individuals that have facial hair.

ATTACHMENT I

Respirator Audit Form

Respirator Audit Form

The purpose of the following audit form is to determine compliance with the OSHA Respiratory Protection Standard 1910.134(e)4.

Date: ___________________

Person Conduction Audit: _____________________	Clock No. ________

Respirator Wearer: ___________________________	Clock No. ________

Department: ________________________________	Job Classification: ________

Respirator in Use: ___________________________	Cartridge: ________

Operation being performed that mandated respiratory protection and hazard involved:

- Was the proper respirator and cartridge selected?	Yes 	No

- Did the individual have a fit test conducted within one year?	Yes 	No

- Was the respirator clean?	Yes 	No

- Was the respirator cleaned after use?	Yes 	No

- How was the respirator stored?

- In a sealed plastic bag?	Yes 	No

- In the original box?	Yes 	No

- Was it stored in a clean locker?	Yes 	No

- Was the respirator maintained?	Yes 	No

Please return completed Audit Forms to location EHS Department

ATTACHMETN J

Voluntary Use Form

Information Contained in Appendix D to ' 1910.134 for Employees Using (Dust Mask) Respirators When Not Required Under the Standard

Respirators (dust masks) are effective when properly selected and worn.  However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the user.

Respirator (dust masks) use may provide an additional level of comfort and protection for workers.  Sometimes, workers may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards.

If your employer provides respirators or dust masks for your voluntary use, you need to take certain precautions to be sure that the respirator itself does not present a hazard.

You should do the following:

1.	Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning and care, and warnings regarding limitations.

2.	Choose NIOSH respirators (dust masks) certified for use to protect against the contaminant of concern. A label or statement of certification should appear on the respirator or respirator packaging.  It will tell you what the respirator is designed for and how much it will protect you.

3.	Do not wear your respirator (dust mask) into atmospheres containing contaminants for which your respirator (dust mask) is not designed to protect against.  For example, a respirator designed to filter dust particles will not protect you against gases, vapors, or fumes or smoke.

4.	Keep track of your dust mask so that you do not mistakenly use someone else's.

Employee Signature						Date

Management Signature						Date

Attachment K – Sample Fit Test Record

Sample Respirator Fit Test Record

Name: __________________________________________________ Initials: ________

Type of qualitative/quantitative fit test used: _________________________________

Name of test operator: _____________________________________ Initials: _______

Date: _________________

Respirator Mfr./Model/Approval no.		Size	Pass/Fail  or	Fit Factor

Note: “Fit factor” is numerical result of quantitative fit test from instrument reading

1. _______________________________________S M L	    P    F	  _____

2. _______________________________________S M L	    P     F	  _____

3. _______________________________________S M L	    P     F	  _____

4. _______________________________________S M L	    P     F	  _____

Clean Shaven?	  Yes___	No___ (Fit-test cannot be done unless clean-shaven)

Medical Evaluation Completed?	Yes___	No___

NOTES: _____________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

This record indicates that you have passed or failed a qualitative or quantitative fit test as shown above for the particular respirator(s) shown.  Other types will not be used until fit tested.

ATTACHMENT L

Definitions

Air purifying respirator (APR) – a type of respirator that removes specific contaminants from air by use of filters, cartridges or canisters by passing ambient air through the air-purifying element.  APRs do not supply oxygen.

Allowable limit – the maximum concentration of a substance in air that is permitted by regulation or voluntary standards to protect employee health.  These concentrations may be expressed in terms of an 8-hour time-weighted average, a 15-minute short term average or as an instantaneous upper ceiling limit. An example is the OSHA permissible exposure limits (PEL).

Assigned protection factor – the level of respiratory protection expected to be provided by a given class of respirators to a properly fitted and trained user.  This factor is assigned by OSHA in substance specific standards and by ANSI in the voluntary national standard, Z88.2.

Atmosphere-supplying respirator – a type of respirator that supplies the user with breathing air from a source independent of the ambient atmosphere, and includes supplied-air respirators (SARs) and self-contained breathing apparatus (SCBA) units.

Canister or cartridge – a container with a filter, sorbent or catalyst, or combination of these items, which removes specific contaminants from the air passed through the container.

Demand respirator – an atmosphere-supplying respirator that admits breathing air to the facepiece only when a negative pressure is created inside the facepiece by inhalation.

Dust mask – see Filtering facepiece.

Emergency situation – any occurrence such as, but not limited to, equipment failure, rupture of containers, or failure of control equipment that may or does result in an uncontrolled significant release of an airborne contaminant.

Employee exposure – exposure to a concentration of an airborne contaminant that would occur if the employee were not using respiratory protection.

End-of-service-life indicator (ESLI) – a system that warns the respirator user of the approach of the end of adequate respiratory protection, for example, that the sorbent is approaching saturation or is no longer effective.

Escape-only respirator – a respirator intended to be used only for emergency exit.

Filter or air purifying element – a component used in respirators to remove solid or liquid aerosols from the inspired air.

Filtering facepiece (Dust mask) – a negative pressure particulate respirator with a filter as an integral part of the facepiece or with the entire face piece composed of the filtering medium.

Fit factor – a quantitative estimate of the fit of a particular respirator to a specific individual, and typically estimates the ratio of the concentration of a substance in ambient air to its concentration inside the respirator when worn.

Fit test –use of a protocol to qualitatively or quantitatively evaluate the fit of a respirator on an individual.

Hazardous atmospheres – an atmosphere that contains a contaminant(s) in excess of the allowable limit or contains less than 19.5 percent oxygen.

Hazard ratio – a number calculated by dividing the actual air contaminant concentration by the allowable limit.

Immediately dangerous to life and health (IDLH) – an atmosphere that poses an immediate threat to life would cause irreversible adverse health effect, or would impair an individual’s ability to escape from a dangerous atmosphere.

Loose-fitting facepiece – a respiratory inlet covering that is designed to form a partial seal with the face.

National Institute for Occupational Safety and Health a Federal institute responsible for conducting research and making recommendations for the prevention of work-related illnesses and injuries.  It tests and issues approvals for respirators.

Negative pressure respirator (tight fitting) – a respirator in which the air pressure inside the facepiece is negative during inhalation with respect to the ambient air pressure outside the respirator.

Occupational Safety and Health Administration (OSHA) – the Federal or state agency with authority to issue and enforce workplace health and safety regulations

.

Oxygen deficient atmosphere – an atmosphere with an oxygen content below 19.5 percent by volume.

Physician or other licensed health care professional (PLHCP) – an individual who’s legally permitted scope of practice (i.e., license, registration or certification) allows him or her to independently provide or be delegated the responsibility to provide some or all of the health care services required by this respirator program.

Positive pressure respirator – a respirator in which the pressure inside the respiratory inlet covering exceeds the ambient air pressure outside the respirator.

Powered air purifying respirator (PAPR) – an air-purifying respirator that uses a blower to force the ambient air through air-purifying elements to the inlet covering.

Pressure demand respirator – a positive pressure atmosphere-supplying respirator that admits breathing air to the facepiece when the positive pressure is reduced inside the facepiece by inhalation.

Protection factor – a ratio calculated by dividing the air contaminant concentration outside a respirator by the concentration inside the respirator.  This is measured in a quantitative fit test.

Qualitative fit test (QLFT) – a pass/fail fit test to assess the adequacy of respirator fit that relies on the individual’s response to the test agent.

Quantitative fit test (QNFT) – an assessment of the adequacy of respirator fit by numerically measuring the amount of leakage into the respirator.

Respiratory inlet covering – that portion of a respirator that forms the protective barrier between the user’s respiratory tract and an air-purifying device or breathing air source or both.  It may be a facepiece, helmet, hood, suit or a mouthpiece respirator with nose clamp.

Self-contained breathing apparatus (SCBA) – an atmosphere-supplying respirator for which the breathing air source is designed to be carried by the user.

Service life – the period of time that a respirator, filter or sorbent, or other respiratory equipment provides adequate protection to the wearer.

Supplied-air respirator (SAR) or airline respirator – an atmosphere-supplying respirator for which the source of breathing air is not designed to be carried by the user.

Tight-fitting facepiece – a respiratory inlet covering that forms a seal with the face.

User seal check – an action conducted by the respirator user to determine if the respirator is properly seated to the face.