Procedure

Title:	Work Task Assessment (WTA) Procedure

Procedure Number

Revision:

| Department: Health and Safety Area: Health and Safety | Approved & Released Procedure | Implementation Date : |
| --- | --- | --- |
| Type of Document: Review Period - Days |  |  |

1.0 Purpose:

The purpose of the work task assessment (WTA) is to assess exposures and evaluate health risks. At a minimum the following questions must be answered:

- What are the chemical, physical, and biological agents in the work environment?

- What health effects are associated with excessive exposure to the environmental agents?

- What are the occupational exposure limits (OELs) for each agent?

- How is the work force organized, staffed, and assigned tasks?

- What are the significant sources of exposure?

- What processes, operations, tasks, and work practices pose significant potential for exposure to environmental agents?

- What controls are in place, how are they used, and how effective are they?

# 2.0 Scope:

This procedure defines how to conduct a qualitative work task assessment (WTA).

# 3.0 Responsibilities:

- Production supervisor – provide process knowledge regarding the work place, work force, and job tasks.

- Production employee – provide process, job, and work task specific knowledge.

- WTA Assessor – collect work place, work force and work task information. Review relevant chemical inventory of the processes to assemble environmental agent information. Observe work tasks for each similar exposure group and documents observations on the WTA form.

# 4.0 Definitions:

- Agent – a chemical, radiological, thermal, physical, or biological environmental agent that can cause adverse health effects in an exposed worker .

- Automatic operation - tasks that are performed remotely and do not require direct operator contact or intervention.

- Duration – Length of time. One of three important parameters used to describe the extent and potential consequences of exposures; the other two parameters are frequency and magnitude.

- Engineering controls - remove the hazard or substitute something that is not hazardous. If removal is not feasible, enclose the hazard to prevent exposure in normal operations. Where complete enclosure is not feasible, establish barriers or local ventilation to reduce exposure to the hazard in normal operations.

- Engineering control options – potential engineering control opportunities to reduce exposure.

- Frequency - One of three important parameters used to describe the extent and potential consequences of exposures; the other two parameters are duration and magnitude.

- Magnitude – Intensity (concentration, amount, level, etc) of contact with an environmental agent. One of three important parameters used to describe the extent and potential consequences of exposures; the other two parameters are duration and frequency.

- Manual operation - tasks that require direct operator contact or intervention.

- Process/task history – the historical knowledge about process/task potential for upsets and exposure.

- Qualitative Exposure Rank – a qualitative ranking of potential exposure to a

specific environmental agent.

- Similar Exposure Group (SEG) – A group of workers having the same general exposure profile for an agent because of the similarity and frequency of the task (s) they perform, the similarity of the materials and processes with which they work, and the similarity of the way they perform the task(s).

- Standard Operating Procedure (SOP) - documented processes that a department has in place to ensure services and/or products are delivered consistently every time.

- STEL sampling – short term exposure limit (STEL) sampling techniques are used to quantitatively assess exposures during specific operations and tasks.

- Source of Exposure – A worker’s contact with an environmental agent via inhalation (air), skin contact (dermal), ingestion, and through migration pathways.

- Task – A work element of series of work elements

- Training information – documentation of worker training and education regarding environmental agents involved with specific processes and tasks.

- Work practice control options – potential work practice control opportunities to reduce exposure.

- Work Task Assessment (WTA) – is a qualitative collection of information to understand the work place, work force, environmental agents, health risks, and associated potential for exposure.

# Procedure:

    - Basic Characterization – A thorough characterization of the work place, the work force, and the environmental agents is needed before various qualitative and quantitative tools can be used to assess occupational exposures.

      - Gather work place information – through interviews with production supervision, workers & engineering staff document the work place regarding site specific processes. Process schematics and flow diagrams are particularly helpful in characterizing continuous, semi-continuous, and batch operations. Collect information to document unit operations and work areas with exposure potential,

and provide details of production activities and process chemistry.

      - Gather work force information – The work force characterization is based on a variety of resources, including plant roster, job descriptions, worker interviews, management interviews, and most importantly, careful observation of work practices and the work place. through interviews with production supervision, workers, and engineering staff document the division of labor, job descriptions/categories, work practices. Acquire knowledge of

routine and non-routine tasks.

      - Gather information on environmental agents – Identify each potentially hazardous chemical, physical, and biological agent in the work place. For each agent, information describing its use, physical properties, routes of exposure, potential health effects, and pertinent OELs should be gathered. Interviews with production

supervision and review of the site’s chemical inventory of hazardous materials and associated Safety Data Sheet (SDS) provide a good source for information. Categories for environmental agents include:

        - Raw materials

        - Intermediates

        - Products

        - Byproducts

        - Additives

        - Maintenance and construction materials

        - Laboratory chemicals

        - Hazardous waste

        - Physical agents

        - Biological agents

      - Establish similar exposure groups (SEG) - Similar Exposure Groups are established by observation using data gathered during the

basic characterization of the work place, work force, and environmental agents. Group work/jobs into SEGs based on their similarities and frequencies of the tasks they perform and their potential exposure to environmental agents.

    - Conduct the Work Task Assessment (WTA): Use the WTA form as the basis for the collection and documentation of task based observations for each SEG. The WTA form is also used to rank in priority potential task specific opportunities for improvement in engineering and work practice control implementation. Work Task Assessment steps:

      - Identify Business Unit, Production Area, Cost Center, Process and Job Title.

      - Ask production supervisor or lead if an SOP exists for the job being assessed. If yes, obtain a “non-controlled” copy.

      - Ask production supervisor or lead if job specific health and safety training is provided to workers performing the job. If yes,

obtain documentation if available.

      - Identify WTA Assessor and the Date in the spaces provided.

      - Explain the purpose of the WTA to the worker and indicate to him/her that you will be asking questions during the assessment.

      - Observe work tasks for the selected SEG; document the tasks in the spaces provided on the form; note whether the task is manual

or automated; be sure to inquire about the workers knowledge of process history (e.g. equipment failures and/or process upsets) and non-routine tasks.

      - Observe and inquire about the length of time the task takes and the frequency that it occurs.

      - Document the Source of Exposure to skin, air, clothing, migration. Pay attention to specific migration pathways on work boots,

PPE, tools, equipment, etc.

      - Document the environmental agent(s) involved with the task.

      - Describe the exiting Engineering Controls (e.g. full or partial enclosure, local exhaust ventilation and Work Practice Controls.

        - If local exhaust ventilation (LEV) exists measure and document face velocities where appropriate.

        - Make note of the ACGIH LEV design criteria for the specified process engineering control.

        - Make note of PPE being used for the specific task.

      - Document whether the task is a candidate for Short Term Exposure Limit (STEL) sampling.

      - Rank order 5 priority tasks having potential “beryllium” exposure improvement opportunities.

      - Note the priority ranked 5 tasks on the back of the form and describe the potential “beryllium” exposure opportunities for improving engineering and work practice controls.

  - Safety & Environmental Information

    - The WTA assessor must wear the same PPE as the employee during the observation process.

  - Associated Documents

    - WTA form

  - References

    - A Strategy for Assessing and Managing Occupational Exposures; American Industrial Hygiene Association (AIHA), Third Addition, Ignacio and Bullock, 2006.

# 6.0 Safety & Environmental Information:

7.0 Associated Documents:

External Documents

BW-ELM-FOR-0371 - Work Task Assessment (WTA) Template Associated Quality System Documents - None

# 8.0 Document Revision History:

| Revision: 1 | Date Created: Date of Last Revision: |  | Last Approval Date: |  |
| --- | --- | --- | --- | --- |
| Document Author : |  | Document Manager : |  |  |

9.0 Reason for Change:

| Revision: | Sec/Para Changed | Change Made: | Date |
| --- | --- | --- | --- |

10.0 Notification List:

# 11.0 Approvals:

First Approver's Signature

Name:

Title:

Second Approver's Signature

Name:

Title:

Third Approver's Signature

Name:

Title:

Fourth Approver's Signature

Name:

Title:

Document History Section