Insert Company Logo

Written Exposure Control Plan

OSHA Beryllium Standard

29 CFR 1910.1024

INSERT

Company Name and Address

INSERT

Date Created:

Name:

INSERT

Review Date:

Reviewer Name:

INSTRUCTIONS

WRITTEN EXPOSURE CONTROL PLAN DEVELOPMENT

In each of the sections below ideas, techniques or procedures are presented in “red” text and offered as examples.  They will aid you in developing your site-specific Written Exposure Control Plan; select from those listed or add your own, as appropriate.  There are also placeholders for you to enter in company specific information.

Upon completion of your Written Exposure Control Plan, all “red” text should be either deleted due to lack of applicability or changed to “black” text due to your review and acknowledgement that the idea, technique, or principle applies to your operations and that you have customized the text to align with site-specific implementation.

This template has been prepared using information and data from sources considered to be technically reliable and is believed to be correct.  Materion makes no warranties, expressed or implied, as to the accuracy of the information contained herein.  Materion cannot anticipate all conditions under which this information and the subject products may be used and the actual conditions of use are beyond its control.  The user is responsible for evaluating all available information when using the subject product for any particular use and complying with all federal, state, provincial and local laws, directives, statutes and regulations.

| Introduction |  |
| --- | --- |

INSERT COMPANY NAME processes beryllium-containing materials in manners that meet the scope and application section of 29 CFR 1910.1024 Occupational Safety and Health Administration (OSHA) – Occupational Exposure to Beryllium standard.

Along with a lower Permissible Exposure Limit (PEL) OSHA’s final rule establishes an Action Level (AL), a Short-Term Exposure Limit (STEL) and requires employers who process beryllium containing materials to comply with a number of additional provisions.

| Action Level (AL)1 µg/m3 | Permissible Exposure Limit (PEL)1 µg/m3 | Short-Term Exposure Limit (STEL)2 µg/m3 |
| --- | --- | --- |
| 0.1 | 0.2 | 2.0 |
| µg/m3 – microgram per cubic meter of air 1 calculated as an 8-hour time-weighted average (TWA) 2 as determined over a sampling period of 15 minutes |  |  |

This Written Exposure Control Plan (WECP) will be evaluated at least annually or when changes occur that may affect beryllium exposures (e.g. changes in materials, equipment, personnel, work practices), when new or additional exposures may be occurring, or when there is a medical necessity (i.e. signs or symptoms associated with beryllium exposure).  A copy of this WECP is posted at INSERT LOCATION and is available upon request from INSERT DEPARTMENT NAME.

The accessibility and content of this WECP is shared and discussed in detail with all employees during initial and annual beryllium worker training sessions.

| Written Exposure Control Plan (WECP) |  |
| --- | --- |

This plan identifies the areas/operations having potential beryllium exposure, the exposure controls and work practices our company has implemented along with the personal protective clothing and equipment (PPE) we require our workers to wear.  Our WECP is comprised of a series of lists and procedures as follows:

Lists of:

- Areas and/or jobs where there is a reasonable expectation of:

  - airborne exposure or dermal contact with beryllium

  - airborne exposure at or above the AL

  - airborne exposure above the PEL or STEL

- Engineering controls, work practices and respiratory protection used on each job that processes beryllium-containing materials.

- Required personal protective clothing and equipment for each job that processes beryllium-containing materials.

INSTRUCTION:  An Excel file named Written Exposure Control Plan – Inventory Table Template is available for your use in preparing your site specific documentation of the above lists (link WECP Table Template).  The Inventory Table template must be completed and inserted into Appendix A before proceeding.

Appendix A contains our company’s list of operations with potential beryllium exposure, the engineering and work practices used, and the PPE required for each job.

Procedures to:

- Minimize cross-contamination within beryllium processing areas.

- Keep surfaces as free as practicable of beryllium

- Prevent migration of beryllium from beryllium processing areas to adjacent areas within the workplace or off plant-site.

- Remove, launder, store, clean, repair, and dispose of beryllium-contaminated personal protective clothing and equipment, including respirators

| Procedures for all Beryllium Processing Areas |  |
| --- | --- |

The following site specific procedures, common to all beryllium processing areas are being used, as part of our Written Exposure Control Plan:

- HEPA vacuums and wet methods are used for all cleaning practices in beryllium processing areas.

- Dry brushing or sweeping are prohibited.

- Use of compressed air for cleaning is prohibited unless it is used in conjunction with a ventilation system that adequately captures beryllium particulate.

- All broken skin and cuts shall be covered.

- Hands are washed before eating, drinking, smoking, applying cosmetics, or when they become visibly dirty.

- Designated disposal containers are available and properly labeled for contaminated materials.

- Rotation of workers to control exposure is prohibited.

INSERT ADDITIONAL SITE SPECIFIC PROCEDURES, AS NECESSARY

| Procedures for Minimizing Cross-Contamination |  |
| --- | --- |

INSERT COMPANY NAME uses the following procedures for minimizing the cross-contamination or transfer of beryllium onto walking & working surfaces and work clothing within beryllium processing areas:

- Disposable gloves (e.g. nitrile) are used when handling contaminated materials (including shoes, shoe covers, and respirators).

- Double-gloving techniques are used to facilitate frequent contaminated disposable glove removal/change-out.

- PPE that is visibly contaminated will be changed at the earliest reasonable opportunity.

- Aprons/smocks/disposable impermeable coveralls are used when performing tasks where work uniform contamination is anticipated.

- Rags or wet wipes used for cleaning are disposed of or containerized immediately after use in properly labeled containers.

- Where practicable, equipment, processes and workstations are designated for “beryllium use only” (e.g., a parts cleaner).

- Intercoms/phone are used whenever practicable for communications to lessen foot traffic into beryllium processing areas.

- The use of “personnel cooling fans” are carefully evaluated with respect to placement, use, and directional influence that minimizes disruption of local exhaust ventilation performance; fans are fixed in-place whenever possible and not positioned in the direction of an LEV hood or enclosure.

- Electronic recordkeeping is used to minimize paperwork from entering/exiting beryllium processing areas.

INSERT ADDITIONAL SITE SPECIFIC PROCEDURES, AS NECESSARY

| Procedures for Keeping Surfaces as Free as Practicable of Beryllium |  |
| --- | --- |

INSERT COMPANY NAME uses the following procedures to keep walking and working surfaces in beryllium processing areas, materials designated for recycling, and eating, drinking and smoking areas as free as practicable of beryllium:

- We recognize that preventing the accumulation of beryllium dust and immobilizing and containing beryllium particulate in a controlled manner are essential to minimizing employee beryllium exposures.  Workers are instructed on the importance of exceptional housekeeping practices and proper cleaning techniques for minimizing exposure in beryllium processing areas.

- A cleaning schedule has been established that is followed and audited: after task, daily, weekly, monthly.

- HEPA vacuums and wet methods are used for cleaning.

- Wet floor scrubbers are used to clean beryllium processing area floors per the cleaning schedule.

- Disposable cleaning supplies are used whenever possible (e.g. disposable GOJO® wipes, Swiffers®, sponges, etc.).

- Dual chamber mop and sponge cleaning buckets are used to reduce mop water contamination.

- Walking and working surfaces are constructed and maintained in an “easy to clean” condition.

- The storage of unnecessary items and inventory is minimized within beryllium processing areas.

- Lighting and illumination levels are measured and meet a minimum of 100 lux (i.e. 10 foot candles) in all beryllium processing areas.

INSERT ADDITIONAL SITE SPECIFIC PROCEDURES, AS NECESSARY

| Procedures for Minimizing the Migration of Beryllium from Beryllium Processing Areas to Other Locations Within or Outside the Workplace |  |
| --- | --- |

INSERT COMPANY NAME uses the following procedures for minimizing the migration or transfer of beryllium from beryllium processing areas to other locations within the workplace or off plant-site:

- Disposable nitrile or similar impervious gloves are used, gloves are removed, and hands are washed upon exiting beryllium processing areas.

- Wall-mounted pass-thru openings are used between beryllium processing and adjacent areas to reduce unnecessary worker exposure.

- Intercoms/phones are used for communication to lessen foot traffic into beryllium processing areas.

- Electronic recordkeeping/bar codes are used to minimize paperwork from exiting beryllium processing areas.

- Captive beryllium processing area shoes/boots that remain in a transition area are used to minimize migration of beryllium into adjacent areas.

- Disposable shoe covers are used to reduce contamination to personal shoes/boots of office workers and visitors entering a beryllium processing area.

- Disposable shoe covers are used by workers exiting beryllium processing areas to reduce migration of beryllium into beryllium processing areas.

- Dedicated tools and equipment that remain in beryllium processing areas are used whenever practicable.

- Access to beryllium processing areas is restricted to authorized personnel (signage, physical barrier).

- Transitions zone have been developed at entry and exit from beryllium processing areas (e.g. PPE donning and doffing locations and instructions).

- Tacky floor mats are used at exits and entrances to beryllium processing areas.

- Air showers are used prior to exiting beryllium processing areas.

- Segregated “clean/dirty” locker rooms are uses for personal clothing/clean PPE/work clothing and contaminated PPE/ work clothing.

- Contractors and visitors are instructed and will follow all PPE, housekeeping, and migration control procedures associated with this WECP.

- All equipment prior to leaving the beryllium processing areas for repair or servicing shall be cleaned to remove visible contamination.

- Material/worker flow is evaluated to prevent contaminated materials or people from moving outside beryllium processing areas.

- HVAC, LEV, and air make-up systems are evaluated and adjusted to maintain a slight negative air pressure (e.g. - 0.05” WG) into beryllium processing areas.

- Dust is HEPA vacuumed from shipping containers.

- Disposable liners are used to prevent contamination of containers (e.g. waste, recycle, and shipping containers).

- Contaminated PPE or work clothing is prohibited from leaving beryllium processing areas into lunch/break rooms, offices, control rooms and other common areas.

- Aprons/smocks are used by office personnel and visitors when entering a beryllium processing area.

- The use of woven materials (e.g., shop rags, belts, shoe strings, cloth gloves, cloth chairs, carpets and floor mats, etc.) which have the potential to collect beryllium particulate is minimized.

- Cameras are used to remotely monitor processes to minimize worker traffic into beryllium processing areas or contaminated areas.

INSERT ADDITIONAL SITE SPECIFIC PROCEDURES, AS NECESSARY

| Procedures for removing, laundering, storing, cleaning, repairing, and disposing of beryllium-contaminated personal protective clothing and equipment, including respirators |  |
| --- | --- |

INSERT COMPANY NAME uses the following procedures for controlling beryllium exposure during the removal, laundering, storage, cleaning, repairing, and disposal of beryllium-contaminated PPE.

Example considerations for handling PPE:

- Disposable PPE is used whenever possible to minimize laundering needs.

- Nitrile or similar impervious gloves are used whenever cleaning equipment and PPE to remove beryllium contamination.

- Disposable wet wipes (or similar) are used to clean respirators, face shields, etc.

- Respiratory protection is always stored in a sealed bag or closed container when not in use.

- Disposable personal protective equipment garments (e.g. coveralls, gloves, booties, etc.) are removed by turning them inside out to help contain contamination.

- Employees will remove all beryllium-contaminated personal protective clothing and equipment at the end of the work shift, at the completion of all tasks involving beryllium, or when personal protective clothing or equipment becomes visibly contaminated with beryllium, whichever comes first.

- Employees will store and keep beryllium-contaminated personal protective clothing and equipment separate from street clothing to prevent cross-contamination.

- Change rooms are provided and used when employees are required to use personal protective clothing or equipment to prevent dermal contact with beryllium.

- A written notice is sent to all vendors who launder/repair beryllium-contaminated clothing and equipment informing them of beryllium contamination.

INSERT ADDITIONAL SITE SPECIFIC PROCEDURES, AS NECESSARY

APPENDIX A

INSTRUCTION: An Excel file called Written Exposure Control Plan – Inventory Table Template is available at http://uat-materion-bsafety.tpgcloudapp.com/resources.  Download the file and complete the form template with your site and job specific details and attach under Appendix A.  Example on following page: